ICOD printer dynamic depot function

(C language statement format)

//Function：connect print equipment

//parameter: iport – equipment communication interface number，for example COM1 - 1 COM2 - 2   etc；the details please see the end of the document

//parameter：baud – baud rate 9600 , 38400 , 115200

//returned value：0 -- success   -1  --  fail

int __stdcall YkOpenDevice(int iport,int baud);

// Function：switch off print equipment

//parameter：no

//returned value：0 --success   -1  --  fail

int __stdcall YkCloseDevice();

// Function：get equipment operation handle

//parameter：no

//returned value：>0 – handle value   -1  --  fail

int __stdcall YkGetDeviceHandle();

////general function

/*

Function description：set character

Variable name			type		variable meaning		remarks

Suction parameter：

iInDoubleHieght		int			double height

iInDoubleWide		int			double width

iInCharB		int			character B		default height and width：

0:each line character

1:each line character

Returned value：					int						0:success

1:fail

*/

int __stdcall SetFont(int iInDoubleHeight,int iInDoubleWide,int iInCharB);

/*

Function description：set left distance

Variable name			type		variable meaning		remarks

Suction parameter：

iInDistance			int			left distance unit		unit：.1mm

Returned value：					int							0: success

1:fail

*/

int __stdcall SetLeftDistance(int iInDistance);

/*

Function description：set line height

Variable name			type		variable meaning		remarks

Suction parameter：

iInDistance			int			line distance		 unit：.1mm equal to default height.75mm

Returned value：					int							0:success

1:fail

*/

int __stdcall SetRowDistance(int iInDistance);

/*

Function description：print bit image LOGO

Variable name			type		variable meaning		remarks

Suction parameter：

pcInBmpFile			char*		Bmp file name		support relative path and absolute支持path，picture take black and white bmp format(a point with one bit),resolution around *100,szie is less than k

Returned value：					int							0:success

1:fail

*/

int __stdcall PrintBmp(char* pcInBmpFile);

/*

Function description：print character

Variable name			type		variable meaning		remarks

Suction parameter：

pcInstring			char*		character need to print		not auto linefeed

Returned value：					int							0:success

1:fail

*/

int __stdcall PrintString(char* pcInstring);

/*

Function description：print character

Variable name			type		variable meaning		remarks

Suction parameter：

pcInstring			char*		one line character need to print		auto linefeed

Returned value：					int							0:success

1:fail

*/

int __stdcall PrintLine(char* pcInstring);

/*

Function description：paper cut

Variable name			type		variable meaning		remarks

Suction parameter：

iInCutMode			int		cut type		0:full cut 1:partial cut

Returned value：					int							0:success

1:fail

*/

int __stdcall CutPaper(int iInCutMode);

/*

Function description：set black mark paper cut position

Variable name			type		variable meaning		remarks

Suction parameter：

iInn				int			black mark paper cut position		unit:0.1mm n=0:paper切cut to black mark position n>0 paper cut under black mark n<0 paper cut above black mark

Returned value：					int							0:success

1:fail

*/

int __stdcall SetBlackMark(int iInn);

/*

Function description: feed paper

Variable name			type		variable meaning		remarks

Suction parameter：

iInDistance			int			0.1mm		单位:0.1mm

Returned value：					int							0:success

1:fail

*/

int __stdcall FeedPaper(int iInDistance);

/****************************************************************/

// Function：Initialize printer 〈detailed command：ESC @〉

//Parameter：no

//Returned value：0 -- success   -1  --  fail

int __stdcall YkInitPrinter();

// Function：send the character string will be printed to printer buffer(note: reach to full line will be printed automatically)

//parameter：pstr – will send character string to data buffer，len – character string data length

//Returned value：0 -- success   -1  --  fail

int __stdcall YkPrintStr(char *pstr);

// Function：print and Enter, but no feed paper〈detailed command：CR〉

//Parameter：no

//Returned value：0 -- success   -1  -- fail

int __stdcall YkEnter();

// Function：print and change one line, feed paper to next line〈detailed command：LF〉

//Parameter：no

//Returned value：0 -- success   -1  --  fail

int __stdcall YkFeedPaper();

// Function：under page mode, cancel printing data〈detailed command：CAN〉

//Parameter：no

//Returned value：0 -- success   -1  --  fail

int  __stdcall YkCancelPrintInPM();

// Function：real-time response host PC request〈detailed command：DLE ENQ n〉

//Parameter：n=1：recover from error status，print from the line of error status.

//Parameter：n=2：clear away receiving buffer area，print buffer area content.

//Returned value：0 -- success   -1  --  fail

int __stdcall YkResponse(int n);

// Function：Tab horizontal positioning, print from the next horizontal position〈detailed command：HT〉

//Parameter：no

//Returned value：0 -- success   -1  --  fail

int __stdcall YkTabMove();

// Function：under page mode, print data〈detailed command：ESC FF〉

//Parameter：no

//Returned value：0 -- success   -1  --  fail

int __stdcall YkPrintInPM();

// Function：set western language right distance，the least bit as set unit〈detailed command：ESC SP n〉

// Parameter：n = 0~255 ,default = 0

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetCharRightSpace(int n=0);

// Function：set character print way〈detailed command：ESC ! n〉

//Parameter：n = 0~255 ,default = 0

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetFontStyle(int n=0);

//Function：set absolute print position〈detailed command:ESC $ nL nH〉

//Parameter：nL – position value low byte, nH – position value high byte, 0 ≤ (nL + nH × 256) ≤ 65535 (0 ≤ nL ≤ 255 , 0 ≤ nH ≤ 255)

//Returned value：0 -- success   -1  --  fail

int YkSetAbsPrnPos(int nL,int nH);

// Function：Enable/Forbidden user defined character〈detailed command:ESC % n〉

//Parameter：n = 0~255 ,default = 0

//Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableUserDefineChar(int n=0);

// Function：Set user defined character〈detailed command：ESC & y c1 c2 [ x1 d1...d(y×x1)]...[ xk d1...d(y×xk)]〉

//Parameter：c1, c2 separately is start code and end code，at most 95 pieces。

//Parameter：character width，dot as unit，decided by character type

//Parameter：code –user-defined internal code of character，for example：use Font B, defined spacing as black patch Internal code is c1=0x20，c2=0x20，code[0]=0xff，code[1]=0xff...code[26]=0xff

// YkSetUserDefineChar(0x20,0x20,9,code);

///The details of the following data refer to the user’s manual

//y==3

//0 ≤ x ≤ 12 [Font A (12 × 24)]

//0 ≤ x ≤ 9 [Font B (9 × 17)]

//k = c2 – c1 +1

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetUserDefineChar(int c1,int c2,int x, unsigned char code[]);

// Function：Enabled or forbidden the character with underline function〈detailed command：ESC - n〉

//Parameter：n=1 enabled，n=0 forbidden default n＝0。

//Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableUnderLine(int n=0);

// Function：Set character line spacing to 3.75mm 〈detailed command：ESC 2〉

//Parameter：No

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetDefaultLineSpace();

//Set character line spacing 〈detailed command：ESC 3 n〉

//Parameter：n=0~255, default n=30

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetLineSpace(int n=30);

// Function：Cancel user defined character〈detailed command：ESC ? n〉

//Parameter：n=32~126

//Returned value：0 -- success   -1  --  fail

int __stdcall YkCancelUserDefChar(int n);

// Function：Set horizontal table position〈detailed command：ESC D〉

//Parameter：tabstr --tab positional value make up character string series, for example:

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetTabPos(char * tabstr);

// Function：Enabled or forbidden emphasis print mode〈detailed command： ESC E n〉

//Parameter：n =0 forbidden n=1 enabled

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSetEmphasized(int n);

// Function：Enabled or forbidden overlap print mode 〈detailed command： ESC G〉

// Parameter：n =0 forbidden n=1 enabled

//Returned value：0 -- success   -1  --  fail

int __stdcall YkOverlap(int n);

// Function：Feed paper n dot line after printing (note：can feed paper accurately 1 dot line=0.125mm) 〈detailed command： ESC J n〉

//Parameter：n =0~255

//Returned value：0 -- success   -1  --  fail

int __stdcall YkPrnAndFeedPaper(int n);

// Function：Enter into page mode work 〈detailed command：ESC L〉

//Parameter：No

//Returned value：0 -- success   -1  --  fail

int __stdcall YkEnablePM();

// Function：Choose character font〈detailed command：ESC M〉

//Parameter：n=0 Choose character font A (12×24); n=1 Choose character font B (9×17)

//Returned value：0 -- success   -1  --  fail

int __stdcall YkSelectFont(int n);

// Function：Set the different ASCII character set of eleven countries 〈detailed command：ESC R n〉

//Parameter： n country code

/*

n	 International character set

0	 U.S.A.

1	 France

2	 Germany

3	 U.K.

4	 Denmark I

5	 Sweden

6	 Italy

7	 Spain I

8	 Japan

9	 Norway

10	 Denmark II

11	 Spain II

12	 Latin America

13	 Korea

14	 Slovenia / Croatia

15	 China

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetInterCharSet(int n=0);

// Function：Change page mode to standard mode〈detailed command：ESC S〉

//Parameter：No

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnterSM();

// Function：Under page mode, set print direction〈detailed command：ESC T n〉

//Parameter：n=0~3 ;

/*

n	Print direction		Starting position

0	Left to right		Upper left

1	Bottom to top		Lower left

2	Right to left		Lower right

3	Top to bottom		Upper right

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetDirectionInPM(int n);

// Function：Enabled or forbidden clockwise rotate 90 to print character〈detailed command：ESC V n〉

//Parameter：n=0 forbidden clockwise rotate 90 printing n=1 enabled clockwise rotate 90 printing

// Returned value：0 -- success   -1  --  fail

int __stdcall YkClockwiseD90(int n);

// Function：Under page mode, set print area, the length of printing page range (76~185mm),print width(the max 72mm) 〈detailed command：ESC W xL xH yL yH dxL dxH dyL dyH〉

//Parameter：left print area top left corner x-coordinate top print area top left corner y-coordinate top width  打印区域宽度 height print area width

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetPrnAreaInPM(int left,int top,int width,int height);

// Function：Set relative print position 〈detailed command：ESC \ nL nH〉

//Parameter：nL ,nH ;  actual position = (nL + nH x 256)x0.125 mm

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetRelPrnPos(int nL,int nH);

// Function：Set alignment when printing〈detailed command：ESC  a n〉

//Parameter：n=0 left justifying  n=1 mediate  n=2	right justifying

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetAlign(int n);

// Function：Set out of paper signal of detecting sensor〈detailed command：ESC c 3 n〉

//Parameter：n=0~255 , each definition, refer to user’s manual table instruction

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetPaperSensor(int n);

// Function：Set to stop printing when out of paper〈detailed command：ESC c 4 n〉

//Parameter：n=0~255 , each definition, refer to user’s manual table instruction

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetSensorToStopPrint(int n);

// Function：Enabled or forbidden the switch on printer panel〈detailed command：ESC c 5 n〉

//Parameter：n=0 forbidden n=1 enabled

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnablePanelButton(int n=0);

// Function：Feed paper n character line after printing〈detailed command：ESC d n〉

//Parameter：n character line

// Returned value：0 -- success   -1  --  fail

int __stdcall YkPrnAndFeedLine(int n);

// Function：Set character code table〈detailed command：ESC t n〉

//Parameter：n code table code name data range：0 ≤ n ≤ 5 , 16 ≤ n ≤ 19 , n = 255

/*

n	Character code table

0	Page 0 [PC437 (USA: Standard Europe)]

1	Page 1 [Katakana]

2	Page 2 [PC850 (Multilingual)]

3	Page 3 [PC860 (Portuguese)]

4	Page 4 [PC863 (Canadian-French)

5	Page 5 [PC865 (Nordic)]

16	Page 16 [WPC1252]

17	Page 17 [PC866 (Cyrillic #2)]

18	Page 18 [PC852 (Latin 2)]

19	Page 19 [PC858 (Euro)]

255 Page 255 [User-defined page]

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetCharCodeTable(int n);

// Function：Feed black mark paper to starting position〈detailed command：GS FF〉

//Parameter：No

// Returned value：0 -- success   -1  --  fail

int __stdcall YkFeedToStartPos();

// Function：Set character times    〈detailed command：GS ! n〉

//Parameter：hsize horizontal enlargement times vsize vertical enlargement times, data range 0~7

/*

hsize ,vsize Value and times relative relationship

0	1 times（original size）

1	2 times

2	3 times

3	4 times

4	5 times

5	6 times

6	7 times

7	8 times

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetCharSize(int hsize,int vsize);

// Function：Under page mode, set absolute vertical printing starting position in print area〈detailed command：GS $ nL nH〉

//Parameter：nL nH positional value  position = (nL + nH x256) x 0.125 mm

// Returned value：0 -- success   -1  --  fail

int YkSetAbsVertPosInPM(int nL,int nH);

// Function：Do test print        〈detailed command：GS ( A pL pH n m〉

//Parameter：n,m

/*

n specify paper source when testing

0	Basic sheet (roll paper)

1	Roll paper

2	Roll paper

m specify test sample

1	Hexadecimal dump print

2	Printer status print

3	Rolling pattern print

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDoTestPrint(int n,int m);

// Function：Enter or exit user set modes 〈detailed command:GS ( E pL pH 〉

//Parameter：m=1 Enter  m=2 Exit

// Returned value：0 -- success   -1  --  fail

int __stdcall YkUserDefCmd(int m);

/ Function：Set MemorySwitch                   〈detailed command:GS ( E pL pH 〉

//Parameter：n=0~7 MemorySwitch 1～8。

//Parameter： memory[] memory1~8 each switch

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetMemorySwitch(int n, int memory[]);  ///???

// Function：Read MemorySwitch     〈detailed command:GS ( E pL pH〉

//Parameter：n=0~7 MemorySwitch 1～8。

//Parameter：memory[]memory1~8 each switch

// Returned value：0 -- success   -1  --  fail

int __stdcall YkGetMemorySwitch(int n,unsigned char memory[11]); ///???

/ Function：Set paper cut and start printing position of black mark〈detailed command:GS ( F pL pH a m〉

//Parameter a =1 set starting position set value a=2 set start paper cut

//Parameter m = 0 specify feed paper direction m = 1 specify reverse paper feed direction

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetBlackMarkParam(int a,int m,int nL,int nH);

// Function：(Note：different function according to work status)〈detailed command：FF〉

//	page mode：return to standard mode after printing

//	black mark mode：set black mark to start position after printing打印后走黑标到打印起始位置。

//Parameter：No

// Returned value：0 -- success   -1  --  fail

int __stdcall YkPrnAndBackToStd();

// Function：Customized printer control value  〈detailed command:GS ( M pL pH a n m〉

//Parameter：

/*

Save or download the defined data.

n	Function

1	Send command GS ( F data and save to NV storage

2	Download GS ( F from NV storage

3	Specify forbidden or automatic allow download data

m specify data as below：

m = 0	the same as GS ( F

m = 1	will be save storage

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDefineControl(int n,int m);

// Function：Set barcode print position of HRI character           〈detailed command:GS H n〉

//Parameter：n=0 not print,n=1 up barcode,n=2 below barcode,n=3 up and below barcode

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetHRIPos(int n=0);

// Function：Read printer ID         (detailed command:GS I n〉

//Parameter：n=1~3   different model different content, please check user’s manual

/*

n	printer ID

1	printer model ID

2	type ID see user’s manual table

3	hardware version ID

*/

//Returned value：0 -- success   -1  --  fail

int __stdcall YkGetPrinterID(int n);

// Function：Set left margin           〈detailed command:GS L nL nH〉

//Parameter：nL nH  left margin =(nL + nH x 256) x 0.125mm

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetLeftMargin(int nL,int nH);

// Function：Set print position to the start of line    〈detailed command:GS T n〉

//Parameter：n=0 after deleting all data in buffer area, set print position to the start of line n=1 after printing all data in buffer area, set print position to the start of line

//Returned value：0 -- success   -1  --  fail

int __stdcall YkToLineHome(int n);

// Function：Execute paper cut action，include feed paper    〈detailed command：GS V m〉

//Parameter：m=66 n: printer feed paper to (paper cut position+ [n × 0.125mm{0.0049inch}])and cut paper，usually n=0

// Returned value：0 -- success   -1  --  fail

int __stdcall YkCutPaper(int m,int n);

// Function：Set print area width       〈detailed command：GS W nL nH〉

//Parameter：nL –print area width low byte nH – print area width high byte

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetPrnAreaWidth(int nL,int nH);

// Function：Under page mode, set vertical print starting position relative the present position〈detailed command：GS \ nL nH〉

//Parameter：nL –position low byte nH –position high byte

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetRelVertPosInPM(int nL,int nH);

// Function：Enable or forbidden ASB        〈detailed command：GS a n〉

//Parameter：0 -- forbidden  1 -- enable

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableASB(int n);

// Function：Enable or forbidden smoothness mode       〈detailed command：GS b n〉

//Parameter：0 -- enable  1 -- forbidden

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableSmoothPrn(int n);

// Function：Set HRI character type of barcode        〈detailed command：GS f n〉

//Parameter：0 – font A (12 × 24)  1 – font B (9 × 17)

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetHRICharStyle(int n);

// Function：Set barcode height             〈detailed command：GS h n〉

//Parameter：n=1~255  the dots in vertical direction

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetBarCodeHeight(int n);

// Function：Print barcode                  〈detailed command：GS k m n d1 ... dn〉

//Parameter：m=65~75

/*

m	barcode system		character number			note

65	UPC-A			11 ≤ n ≤ 12		48 ≤ d ≤ 57

66	UPC-E			11 ≤ n ≤ 12		48 ≤ d ≤ 57

67	JAN13 (EAN13)	12 ≤ n ≤ 13		48 ≤ d ≤ 57

68	JAN8 (EAN8)		7 ≤ n ≤ 8			48 ≤ d ≤ 57

69	CODE39			1 ≤ n ≤ 255		48 ≤ d ≤ 57, 65 ≤ d ≤ 90, 32, 36,37, 43, 45, 46, 47

70	ITF				1 ≤ n ≤ 255 (n is even number) 48 ≤ d ≤ 57

71	CODABAR			1 ≤ n ≤ 255		48 ≤ d ≤ 57, 65 ≤ d ≤ 68, 36, 43,45, 46, 47, 58

72	CODE93			1 ≤ n ≤ 255		0 ≤ d ≤ 127

73	CODE128			1 ≤ n ≤ 255		0 ≤ d ≤ 127

74	standard EAN13		12 ≤ n ≤ 13		48 ≤ d ≤ 57

75	standard EAN8		7 ≤ n ≤ 8			48 ≤ d ≤ 57

*/

//Parameter：n character number of the above table

//Parameter：barcode the data need to change to barcode

// Returned value：0 -- success   -1  --  fail

int __stdcall YkPrintBarCode(int m,int n,char * barcode);

// Function：Get printer status   〈detailed command：DLE EOT  n〉

//Parameter：n = 1~5

// Returned value：0 -- success   -1  --  fail

int __stdcall YkGetStatus(unsigned char n);

// Function：Read printer status                   〈detailed command：GS r n〉

//Parameter：n=1 transmit paper sensor status

// Returned value：0 -- success   -1  --  fail

int __stdcall YkGetPrinterStatus(unsigned char n);

// Function：Set barcode width            〈detailed command：GS w n〉

//Parameter： n=2~6

/*

Dual barcode

n		multi element barcode unit width (mm)	fillet width (mm)		wide width(mm)

2		0.250						0.250				0.625

3		0.375						0.375				1.000

4		0.560						0.500				1.250

5		0.625						0.625				1.625

6		0.750						0.750				2.000

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetBarCodeWidth(int n);

//Function：Set Chinese character print mode group〈detailed command：FS ! n〉

//Parameter：n=0~255 See command table

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetChineseFontStyle(int n);

// Function：Enter Chinese character mode〈detailed command：FS &〉

//Parameter：no

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableChineseMode();

// Function：Exit Chinese character mode〈detailed command：FS .〉

//Parameter：no

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDisableChineseMode();

// Function：Enable or forbidden Chinese character with underline mode〈detailed command：FS - n〉

//Parameter：n=0 forbidden n=1 enable

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableChineseUnderLine(int n);

// Function：Specify user defined Chinese character 〈detailed command：FS 2 c1 c2 d1...dk〉

//Parameter：c1、c2 Zone bit code in the Chinese character user defined area, see user’s manual table, for example:  GB18030,c1 = FE A1 ≤ c2 ≤ FE  ,k=72, d range 0~255

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetUserDefChinese(int c1,int c2,int code[]);

// Function：Set user defined character code system    〈detailed command：FS C n〉

//Parameter：user defined character code system n= 0~1

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetUserDefChineseArea(int n);

// Function：Set Chinese character left and right spacing   〈detailed command：FS S n1 n2〉

//Parameter：left spacing n1,right spacing n2,0 ≤ n1 ≤ 255,0 ≤ n2 ≤ 255

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetChineseLeftRightSpace(int n1,int n2);

// Function：Enable or forbidden Chinese character quadruple print 〈detailed command：FS W n〉

//Parameter：n=0 forbidden n=1 enable

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetChinese4TimesSize(int n);

// Function：Enable or forbidden reverse print mode <detailed command: ESC { n〉

//Parameter：n=0 forbidden n=1 enable

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableUpsidedown(int n);

// Function：Print reserve definition bit image      〈detailed command: FS p n m〉

//Parameter：m print density

/*  m	result	vertical density (dpi)	horizontal density (dpi)

0	common	203				203

1	double width	203				101

2	double height	101				203

3	4 times		101				101

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkPrintDefineBitmap(int m);

// Function：Download defined bit image      〈detailed command: FS q n〉

//Parameter：szBmpFile

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDefineBitmap(char *szBmpFile);

// Function：Immediately print BMP    〈detailed command:ESC * m nL nH d1...dk 〉

//Parameter：szBmpFile。

m		vertical density	horizontal density

0		60			90

1		60			180

32		180			90

33		180			180

generally m=33

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkPrintBitmap(char *szBmpFile,int m);

// Function：Download image and print immediately       〈detailed command：firstly execute GS * ，then execute GS / m〉

//Parameter：szBmpFile

/*

m	result	vertical density(dpi)	horizontal density(dpi)

0	common	180			180

1	double width	180			90

2	double height	90			180

3	4 times	90			90

generally m=0

*/

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDownloadBitmapAndPrint(char *szBmpFile,int m);

/// Function：Set cash drawer mode    〈detailed command： ESC p m t1 t2〉

//Parameter：  m=0  2pin  m=1  5pin    pulse width: t1*2ms t2*2ms ， t1 < t2 ，generally t1=150 t2=250

// Returned value：0 -- success   -1  --  fail

int YkSetCashBoxDriveMode(int m,int t1,int t2);

// Function：Set one status callback function(note: suitable for all printer model with status feedback, for example: serial)

//Parameter：pCallBack

// Returned value：0 -- success   -1  --  fail

int __stdcall YkSetCallBack(CallBack pCallBack);

//Function：Enable callback function

//Parameter：enable = 1 ，Enable =0 forbidden

// Returned value：0 -- success   -1  --  fail

int __stdcall YkEnableCallBack(int enable=0);

// Function：Communicate with equipment data directly

//Parameter：pdata – will send printer buffer data，len – data length

// Returned value：0 -- success   -1  --  fail

int __stdcall YkDirectIO(unsigned char *pdata,int len);

///Serial port

#define COM1	1

#define COM2	2

#define COM3	3

#define COM4	4

#define COM5	5

#define COM6	6

#define COM7	7

#define COM8	8

#define COM9	9

#define COM10	10

///Parallel port

#define LPT1	11

#define LPT2	12

///USB

#define USB		13

///(wired or wifi)network printer

#define NET		14